Name ___________________________________

R111

Computer Aided Manufacturing

G-Clamp Go-no-go Gauge

(for Quality Control)

2D Design learning

- Drawing 2D Shapes

- Tracing shapes

Keyfob Laser Cutting Demo & notes

Laser Cutting notes (these are important – make thorough notes):

2D Design settings

Safety and Machine Checks

Machine Setup and operation

Produce your own Acrylic Keyfob

Acrylic Keyfob (60 x 60mm limit).

Use at least 2 depths (cutting/engraving)

Batch produce 2.

Hex 2D Design file

Let’s get started
G-Clamp Quality Control
Go-no-go Gauge

The G-Clamp pre-production prototype produced in unit R110 has been approved for manufacture. As part of the Quality Control procedures on the production line, a Go-no-go Gauge is to be produced for use by workers on the production line. It will be used to ensure that the G-Clamps coming off the production line are within accepted tolerances.

A small batch quantity of five Go-no-go Gauges is to be manufactured using CNC machining.

For this unit, you will need to:

- interpret the engineering drawing provided (later in this booklet)

- produce a CAD drawing of the component to be manufactured suitable for export to the CNC machine

- export drawing information to CNC and perform on-screen simulation of production interpreting and acting upon the information gained

- produce a detailed production plan for the batch manufacture of the Go-no-go Gauge

CNC Production Plan

In order to be successful, the production must be carefully planned. Produce a detailed plan for the batch manufacture of the Go-no-go Gauge.

It must include:

- Steps in the correct order

- Description of how to do each step

- Safety (using laser cutter)

It should include:

- Tool changes – different colours in line and fill for different laser operations

- Material - setting correct speeds for cutting and engraving etc

- Tool offsets - Setting the laser starting point, depth

- Scale – how to produce more than one

- Timing – How long each step will take

It could include

- Quality Control – How you will ensure each step is checked

Example:

G-Clamp Go-no-go Gauge
Dimension drawing

Shaded area denotes engraved area

Annotated Dimension drawing

TASK

- Study every aspect, every part, every dimension.

- Annotate the materials

- Calculate and annotate the measurements that are not explicitly provided on the drawing

- Annotate, showing what cutting operation / speed/power settings might be needed for different parts of the drawing

LO2: Screenshot Evidence

Drawing (2D Design)

Screen shots of you drawing the Go-no-go Gauge

Pasted into your R111 shared document and annotated

Example:

LO2/LO3: Screenshot Evidence 
Settings for Laser Cutting

Annotated photos of you operating the
laser cutter

- Setting up on the laser cutter computer

- Setting up the laser cutter

- Cutting

Example

LO3: Photo Evidence 
Laser Cutter setup, Simulation, Operation

LO3 optional: Review of Quality

To access higher marks, review the quality of your finished piece against the Dimension Drawing. An example of how you could do this is here:

LO3 optional: Risk Assessment

What is a risk assessment?

‘Risk assessment’ is a term used to describe a process or method where you:

- Identify hazards (risks and dangers) that could cause harm

- Evaluate the level of risk associated with that hazard (how likely it is to happen and how serious the effect if it did happen).

- Decide precautions to reduce the risk of them happening (what can we do to reduce the chances or remove the risk entirely)

- Record this for others to refer to and use

- Review it in the future

Your Risk Assessment

You need to use the laser cutter for this project. Produce a risk assessment for using the laser cutter.

This could be done in a table (decide on suitable headings yourself)

Suggested steps:

- List the hazards / dangers

- For each one:

  - Decide how likely each is to happen (perhaps with a number range?)

  - Decide how serious each is if it did happen

Record actions to reduce the risk

LO3: Manual vs. CNC Comparison

Manually Made Checker

You will now build a version of the Go-no-go Gauge by hand. Use the dimension drawing, measuring tools and cutting and filing tools.

You will then write a comparison between the laser cut version and this manual version.

Comparison

If you made a batch of the checkers with CNC and by hand, there would be some big differences.

Mention the differences in consistency, cycle time and visual finish

Example:

LO4: Report

You need to produce evidence that shows you know about applications of computer-controlled processes used to manufacture products.

A company wishes to invest in additional computer-controlled equipment in order to improve its manufacturing. The company wishes to be able to work with a range of materials at different scales of production: one-off/prototype, batch and high-volume production.

You have been asked to prepare a report showing different options they could consider investing in.

Consider how rapid prototyping, manufacturing processes and robotics could be used for one-off/prototype, batch and high volume production of a product and its components.

Your report should include:

- An outline of computer-controlled processes/CNC machines used in manufacturing

- Information about processes / machines (such as CNC, additive manufacturing, robotics etc)

- A description with examples of computer-controlled processes used for different scales of manufacture (one-off/prototype, batch and high-volume production).

To access high marks:

Demonstrate knowledge of:

- Types of CNC processes and use of them (with examples)

  - Milling machines, turning centres, fabrication machines

- How different CNC are suited to different production scales (with examples)

  - One off

    - Laminating, 3D printing, stereolithography, laser sintering

  - One off & Batch

    - CNC manufacturing

    - Additive

  - Batch & Mass (high volume)

    - Robotics: welding, riveting, pick and place (fabrication)